- Section 08 62 00 Roof Accessories – Unit Skylights

This document is meant as a spec guide for the discernment of technical specifications that may apply to our products and that may be chosen to apply per a customer requirement. It is not meant as a claim to all the standards listed as applicable to every Series 1400 product sold.

# GENERAL

## SUMMARY

### Section includes:

#### Prefabricated fixed unit skylights

### Related Sections:

#### Section 05050 (05 05 00) - Common Work Results for Metals.

#### Section 05100 (05 10 00) - Structural Metal Framing.

#### Section 05200 (05 20 00) - Metal Joists.

#### Section 05300 (05 30 00) - Metal Decking.

#### Section 05400 (05 40 00) - Cold-Formed Metal Framing.

#### Section 06100 (06 10 00) - Wood Framing. Site built wood curbs and nailers.

#### Section 07310 (07 31 00) - Roof Shingles and Shakes. Flashing of skylight base.

#### Section 07320 (07 32 00) - Roof Tiles. Flashing of skylight base.

#### Section 07360 (07 36 00) - Single Ply Membrane Roofing.

#### Section 07510 (07 51 00) - Built-Up Bituminous Roofing. Flashing of skylight base.

#### Section 07520 (07 52 00) - Modified Bituminous Membrane Roofing. Flashing of skylight base.

#### Section 07525 (07 52 50) - Modified Sheet Roofing.

#### Section 07530 (07 53 00) - Electrometric Membrane Roofing. Flashing of skylight base.

#### Section 07540 (07 54 00) - Thermoplastic Membrane Roofing. Flashing of skylight base.

#### Section 07600 (07 60 00) - Flashing and Sheet Metal. Metal curb flashings.

#### Section 07610 (07 61 00) - Sheet Metal Roofing

#### Section 08620 (08 60 00) - Unit Skylights. Skylights without reflective tube.

## REFERENCES

### Aluminum Association (AA)

#### Specifications for Aluminum Structures.

### American Architectural Manufacturers Association (AAMA)

#### AAMA 1600/IS7, Skylight Standard

#### AAMA Skylight Installation Manual

#### AAMA 1505.1-87, Voluntary Standard Uniform Load Test Procedure for Plastic Glazing Skylights by Uniform Static Air Pressure Difference.

### American Society for Testing and Materials (ASTM)

#### ASTM A463/A463M - Standard Specification for Steel Sheet, Aluminum Coated, by the Hot Dip Process.

#### ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc Coated (Galvanized), by the Hot Dip Process.

#### ASTM B209 - Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate.

#### ASTM B221 - Standard Specification for Aluminum-Alloy Bar, Rod, and Wire.

#### ASTM D412 - Standard Test Methods for Vulcanized Rubber and Thermoplastic Elastomers - Tension.

#### ASTM D635 - Test Method for Rate of Burning and/or Extent of Time of Burning of Self-Supporting Plastics in a Horizontal Position.

#### ASTM D1003 - Standard Test Method for Haze and Luminous Transmittance of Transparent Plastics.

#### ASTM D1929 - Standard Test Method for Determining Ignition Temperature of Plastics.

#### ASTM D2843 - Standard Test Method for Density of Smoke from the Burning or Decomposition of Plastics.

#### ASTM D4802 Specification for Poly (Methyl Methacrylate) Acrylic Plastic Sheet

#### ASTM E84 - Standard Test Method for Surface Burning Characteristics of Building Materials.

#### ASTM E108 - Standard Test Methods for Fire Tests of Roof Coverings.

#### ASTM E283 - Test Method for Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen.

#### ASTM E313 - Standard Practice for Calculating Yellowness and Whiteness Indices from Instrumentally Measured Color Coordinates

#### ASTM E330 - Structural Performance of Exterior Windows, Curtain Walls, and Doors.

#### ASTM E331 Test Method for Water Penetration of Exterior Windows, Curtain Walls and doors by Static Air Pressure Difference.

#### ASTM E1886 - Standard Test Method for Performance of Exterior Windows, Curtain Walls, Doors, and Impact Protective Systems Impacted by Missiles and Exposed to Cyclic Pressure Differentials.

### International Code Council (ICC):

#### International Building Code (IBC 2024)

#### International Energy Conservation Code (IECC 2021)

### North American Fenestration Standard (NAFS):

#### AAMA/WDMA/CSA/101/I.S.2/A440 - North American Fenestration Standard/Specification for Windows, Doors, and Skylights.

### National Roofing Contractors Association (NRCA)

#### Roofing and Waterproofing Manual.

### Factory Mutual (FM)

#### FM 4431 Standard for Skylights

### American Welding Society (AWS)

#### AWS D1.1 Structural Welding Code

## PERFORMANCE REQUIREMENTS

Remove all performance options that are not required.

### High velocity hurricane zones (HVHZ):  Skylights to Meet Missile level D and minimum wind loading of +55/-65psf

### Light-Transmitting Plastics

#### KILON Monolithic or Prismatic polycarbonate

##### Combustion rating:

###### CC1 for Polycarbonate

###### CC2 for Acrylic

##### Smoke Density rating: < 75

##### Smoke-development Index: < 450

## SUBMITTALS

### Submit under provisions of Section 01300.

### Product Data: Manufacturer's data sheets on each product to be used, including:

#### Product Data Sheet

#### Installation & Maintenance Manual

#### Installation instructions

### Sales Drawing showing Skylight assembly, base, and accessories

### Certificate of Compliance or third party certificates verifying compliance with performance requirements.

### Closeout Submittals: Warranty Documents

## DELIVERY, STORAGE AND HANDLING

### Handle, deliver and store in manufacturer’s original packaging, following the manufacturer’s recommendations. Store in a dry area and in a manner to prevent damage.

### Exercise caution when hoisting unit to the roof so as not to damage the release mechanism.

## WARRANTY

### Skylight Warranty: Provide written warranty by the manufacturer, agreeing to provide replacement components that exhibits defects in materials or workmanship.

### Warranty period to be 10 years from date of purchase for aluminum framed skylights.   Two years for galvanized steel framed skylights and Galvanized steel base frames.

# PRODUCTS

## MANUFACTURERS

### Acceptable Manufacturer:  Kingspan Light + Air, 28662 N. Ballard Drive, Lake Forest, IL 60045, phone +1 (800) 759 6985, fax +1 (847) 816 0425, http://www.kingspanlightandair.us.

### Substitutions: Not permitted

### Requests for substitutions will be considered in accordance with provisions of Section 01600.

## SKYLIGHTS

Select one of the below skylight options and delete the others.

### SERIES 1400 SKYLIGHT (HWHS)

#### System Description:  Bubble dome skylight with Curb Mount (CM) or Self-Flashing (SF) base.  Constructed of aluminum extruded framing members with fully welded corners with integrated guttering system for condensation management and drainage.

#### Glazing:

[Clear Polycarbonate over White Polycarbonate]

#### Mounting:

##### Curb Mounted (CM)

##### Self-flashing (SF)

###### To be manufacturer’s standard aluminum or galvanized steel,

Select, modify or delete following options as needed for self-flashing base options.

###### [9”] [12”] [_____] high (12” maximum)

###### [1” thick Polyiso insulation (INS)]

[Double Wall insulated (DW)]

[Welded Double Wall Insulated (WDW)]

###### 1” x 4” wood nailer (WN)

###### Termination Bar (TB)

## Accessories

(Optional) Select required accessories below

### Steel Security Bars: Optional steel security/burglar bars by Kingspan Light + Air shall be ½” steel bars welded and assembled into a 1 ½” x 1 ½” x 1/8” steel angle frame and provided with a white powder coated paint finish or approved equal.

### Steel mesh Safety Screen:  Optional steel 0.167” diameter resistance welded wire mesh screen.  Galvanized steel with stainless steel straps to secure to SF base or existing curb.

## FABRICATION

### Fabricated by an ISO 9001:2015 Certified Manufacturer.

### Fabricate free of visual distortions and defects. Weld corners and joints.

### All fasteners and hardware shall be corrosion resistant.

### Provide a weather tight assembly.

# EXECUTION

## INSTALLATION

### Install Skylights in accordance with manufacturer’s instructions. Coordinate roof membrane installation requirements specified in other Sections.

### Skylight curbs bearing on roof deck:

#### Set units in place and secure curb flange to steel roof deck by self-tapping non-corrosive screw fasteners, spaced at a maximum of 12” on center, staggered.

### Apply bituminous paint on metal surfaces of units in contact with cementitious material and dissimilar metals.

### After installation, remove shipping screws and test vents to ensure that vents operate as intended.

END OF SECTION